# Manufacturing/Production Plan

Smart Toolbox

Evan Hoeger, Salem Bird, Walter Blackman, Luke Fredin

## Bill of Materials — Purchased vs. Manufactured Components

Our Bill of Materials consists of a mix of purchased electronic components, purchased hardware, cloud software expenses, and manufactured plastic housings. The purchased components include the Raspberry Pi 5, two Raspberry Pi OV5647 camera modules, a fan with an attached heat sink, and general hardware such as M1×6 screws. These items are purchased because they require technologies, tolerances, or fabrication processes that are impractical or cost-prohibitive to produce ourselves. The Raspberry Pi 5 typically costs around $80 when sourced from Amazon, PiShop, or DigiKey, and quantity discounts can reduce the price to $65–$70 at orders above 100 units. The camera modules cost roughly $32 each from Amazon, with quantity discounts bringing them down to approximately $24 when purchased in bulk. The fan and heat sink cost $5 and can be bulk purchased for $4.25. Hardware such as M1×6 screws are inexpensive commodity items costing about $2.99 per pack, with minimal price change unless purchased at industrial scales. We also rely on AWS S3 cloud storage for data access and management, costing $25 per month for 1.1 TB of storage. Cost efficiency increases with extremely large data volumes, though this is not expected at our scale. In addition, an optional adjustable mounting arm, based on an existing VIVO MIC03L stand, is purchased from Amazon for about $50 and provides a cheaper alternative to developing a custom mechanical solution. Overall, purchased components are selected when external suppliers can provide precision, quality, or economies of scale that far exceed what we could achieve internally.

The manufactured components include the injection-molded ABS shell and lid, which form the main body of the system. These must be manufactured in-house or through a contract manufacturer because they require custom internal geometries, mounting features, and aesthetics that cannot be sourced off-the-shelf. These parts are produced from ABS resin costing $0.064 per cubic inch, with our design totaling roughly eight cubic inches for a material cost of about $0.51. Combined with machine time, the final molded cost is approximately $1.17 per shell assembly. Manufacturing is essential for components that must integrate precisely with our electronics and mechanical layout. Software, while not a physical component, is also internally “manufactured.” Both the face-recognition and tool-recognition systems rely on custom code built on open-source frameworks, allowing us to avoid licensing fees while maintaining full control over system functionality.

Table 1 BOM per box

| Part No. | Part | Quantity | Price | Manufacturing Plan |
| --- | --- | --- | --- | --- |
| 1 | 3D Injection molded Shell (ABS Plastic) | 1 | $1.01 | Contracted Injection Molding |
| # 2 | 3D Injection Molded Shell Lid (ABS Plastic) | 1 | $0.16 | Contracted Injection Molding |
| # 3 | # Raspberry Pi Camera Module 5MP 1080P OV5647 Sensor with M12 FOV90 | 2 | $64 | Purchased in bulk, via communication with Raspberry Pi company |
| 4 | Raspberry Pi 5 | 1 | $80 | Bulk – Raspberry Pi |
| 5 | Raspberry Pi 5 Aluminum Alloy Top Mount Fan with Heat Sink | 1 | $4.25 | Digikey |
| 6 | M1x6 Flat Head Screw | 10 | $2.99 | Fastenal |
| 7 | Face recognition software (Dlib/custom) | 1 | $0 | Open Source |
| 8 | Tool recognition software (Github/custom) | 1 | $0 | Open Source |
| 9 | Back-end Server | - | 25 $ - per company | Purchased through AWS |
| 10 (Optional Attachment) | Mounting Arm | 1 | $49.99 | Amazon |
|  | Total | 1 | $177.41 |  |
|  | Total With Arm | 1 | $227.40 |  |

## Manufacturing Processes, Raw Materials, and Labor Estimates

The primary manufactured components in our system are the ABS plastic shell and lid, which are produced through the injection molding process. These components use ABS pellets as the raw material, which are dried and fed into an injection molding machine. We do not own the injection molding machine; instead, we have the work outsourced to a builder. However, we would need to own the tooling this supplier uses. We plan to use a P20 steel mold costing $12,000–$14,000, which provides a long tool life of 100,000–300,000 units. Each molded part has a cycle time of approximately 30 seconds, allowing for around 120 parts per hour on a standard 120-ton injection molding press. Contract manufacturing rates of $140 per hour result in a machine-time cost of $1.17 per part. The raw material cost for a single shell is just over fifty cents, making labor and machine time the primary cost drivers once full production begins. Manufacturing time depends on the lot size; large batches significantly lower per-unit costs because the fixed mold investment is amortized across a greater number of units. This is why we skip straight to injection molding rather than 3D printing our case. The extra cost vs time saved and quality of end product for injection molding far exceed that of 3D printing.

In addition to physical manufacturing, several software modules also count as internally produced components. These include the face-recognition module, the tool-recognition module, the cloud-integration and database system, and the user configuration interface. Each module involves custom development using Python, machine-learning libraries, and camera-based vision algorithms. The face-recognition module processes camera input to match users to stored profiles, while the tool-recognition module analyzes tool drawer images to detect missing or moved tools. These modules collectively require 150-200 hours of development time. The cloud integration module manages communication between the device and AWS S3 storage, while the user interface provides setup and monitoring tools. Together, the software represents a significant portion of the system’s “manufactured” intellectual content.

## Production Quantity / Price Differential

The most expensive component to manufacture is the primary ABS shell, not because of material cost but because of tooling. During low-quantity prototyping, the shell can be produced using 3D printing at roughly $8–$12 per part, making it appropriate for early development runs below 100 units. As production scales, injection molding becomes more cost-effective. Once we exceed approximately 400–600 units, the per-part savings from molding justify the mold investment. At high volumes, the molded shell costs roughly $1.17 per unit, compared to the much higher 3D printing cost. This transition from additive to injection-molded production significantly reduces unit cost as the business scales, providing clear cost reduction as sales increase.

## Assembly Process, Tools, and Estimated Assembly Times

The majority of assembly is performed by us before the product ships. Internal assembly includes mounting the Raspberry Pi, installing the camera modules, routing wiring, securing the lid, loading firmware, and running diagnostic tests. These steps require only basic tools such as small screwdrivers, a precision bit set, and a computer for software installation. The total internal assembly time is approximately fourteen minutes per unit, resulting in an estimated labor cost of about $7 at a $30 per hour labor rate. These processes ensure that customers receive a fully functional and pre-configured device ready for minimal setup.

Customer assembly is limited to mounting the device on their toolbox and completing initial digital setup. Mounting requires a screwdriver and takes about five to ten minutes depending on whether the customer uses the optional adjustable arm. Power connection and Wi-Fi configuration take just a few minutes and do not require tools beyond a smartphone. By keeping customer-side assembly simple, we improve usability and reduce the risk of incorrect installation. Overall, assembly is divided to maximize efficiency: delicate electronic work is handled by us, while simple mechanical mounting tasks are left to the customer.

## Packaging Plan and Total Out-the-Door Cost

Our packaging plan emphasizes protection, cost-efficiency, and professional presentation. The unit ships in a custom-printed corrugated cardboard box with dimensions around 12×8×6 inches, costing approximately $1.50–$2.50 depending on print volume. Inside the box, a molded pulp tray or foam insert secures the product to prevent damage during transit. This insert typically adds $0.80–$1.20 to packaging cost but significantly reduces damage risk. The package also includes a printed instruction sheet, costing around twenty cents, along with standard labeling and tape. A single-color printed exterior includes branding, a QR code for setup instructions, and compliance information.

With these items combined, the total packaging cost generally falls between $2.60 and $3.90 per unit. When combined with electronics, molded parts, assembly labor, and cloud infrastructure, the total out-the-door cost of each unit is roughly $177. Considering the device sells for $350 plus more for the optional arm, this leaves a workable margin even before revenue from the monthly software subscription is included. The packaging system ensures that each product arrives undamaged, easy to open, and professionally presented.